Hexagon Virtual Machine

Specification [Draft]

80-V9418-14 Rev. A July 8, 2011

Submit technical questions at: https://support.cdmatech.com

Qualcomm Confidential and Proprietary

Restricted Distribution. Not to be distributed to anyone who is not an employee of either Qualcomm or a subsidiary of Qualcomm without the express approval of Qualcomm’s Configuration Management.

Not to be used, copied, reproduced in whole or in part, nor its contents revealed in any manner to others without the express written permission of Qualcomm.

QUALCOMM is a registered trademark of QUALCOMM Incorporated in the United States and may be registered in other countries. Other product and brand names may be trademarks or registered trademarks of their respective owners. ARM is a registered trademark of ARM Limited. Hexagon is a trademark of QUALCOMM Incorporated in the United States and other countries.

This technical data may be subject to U.S. and international export, re-export, or transfer (“export”) laws. Diversion contrary to U.S. and international law is strictly prohibited.

QUALCOMM Incorporated

5775 Morehouse Drive

San Diego, CA 92121-1714 U.S.A.

Copyright ©2011 QUALCOMM Incorporated.

All rights reserved.

Contents

- Introduction

  - Overview ................................................................................................................8

  - Architecture............................................................................................................8

  - Modes.....................................................................................................................9

  - Instructions and events...........................................................................................9

  - Virtual privileged resource model..........................................................................9

  - Using this manual.................................................................................................10

  - Terminology .........................................................................................................10 1.8	Notation................................................................................................................ 11

- HVM User Mode

  - Overview ..............................................................................................................12

  - Restrictions...........................................................................................................12 2.2.1 Memory addressing..............................................................................12

- HVM Guest Mode

  - Overview ..............................................................................................................13

  - Guest mode capabilities........................................................................................14

    - Event handling .....................................................................................14

    - Interrupts..............................................................................................14

    - Exceptions............................................................................................14

    - Cache control .......................................................................................15

    - Memory management ..........................................................................15

    - Processor resource management..........................................................15

  - Restrictions...........................................................................................................16 3.3.1 Memory addressing..............................................................................16

- Initial State

  - Overview ..............................................................................................................17

  - Virtual processor...................................................................................................17

  - Registers...............................................................................................................17

  - Memory ................................................................................................................17 4.5 Initial memory map ..............................................................................................18

- Event Model

  - Overview ..............................................................................................................19

  - Event model..........................................................................................................19

Contents

5.3 Event registers	20

5.3.1 GELR	20

5.3.2 GSR	21

5.3.3 GOSP	21

5.3.4 GBADVA	21

5.4 Event types	22

5.5 Event vector registration	22

5.6 Event handler return	23

5.7 Virtual instructions for event management	23

5.8 Programmers note	23

6 Interrupts

6.1 Overview	24

6.2 Virtual interrupt controller	24

6.3 Interrupt type and polarity	25

6.4 Interrupt masks	25

6.5 Interrupt acknowledgement	25

6.6 Interrupt enable	25

6.7 Virtual instructions for interrupt management	26

7 Exceptions

7.1 Overview	27

7.2 Exception classes	27

7.3 General exceptions	28

7.4 Trap exceptions	29

7.5 Virtual machine check	29

8 Cache Control

8.1 Overview	30

8.2 HVM cache operations	30

8.3 Virtual instructions for cache control	31

9 Memory Management

9.1 Overview	32

9.2 Underlying logical/physical memory	32

9.3 Linear translations	32

9.4 Virtual page table entries	34

9.4.1 Page directory entries	35

9.4.2 Page table entries	35

9.4.3 Cache attributes	37

9.5 Setting new memory maps	38

9.6 Flushing stale memory maps	38

9.7 Virtual instructions for memory management	38

Contents

- Processor Resource Management

  - Overview ..............................................................................................................39

  - Timer ....................................................................................................................39

  - Processor suspension............................................................................................40

  - Processor creation.................................................................................................40 10.5 Virtual instructions for processor management....................................................41

- Trust Model

  - Overview ..............................................................................................................42

- Debug

  - Overview ..............................................................................................................43

  - User-mode software debug...................................................................................43 12.3 Guest-mode software debug.................................................................................43

- HVM Instructions

  - Overview ..............................................................................................................44

  - Instruction properties............................................................................................46

    - VMCACHE..........................................................................................47

    - VMCLRMAP.......................................................................................49

    - VMGETIE............................................................................................50

    - VMINTOP ...........................................................................................51

    - VMGETTIME......................................................................................53

    - VMNEWMAP .....................................................................................54

    - VMRTE................................................................................................56

    - VMSETIE ............................................................................................57

    - VMSETTIME ......................................................................................58

    - VMSETVEC........................................................................................59

    - VMSTART...........................................................................................60

    - VMSTOP .............................................................................................61

    - VMVERSION......................................................................................62

    - VMVPID..............................................................................................63

    - VMWAIT.............................................................................................64

    - VMYIELD...........................................................................................65

- Determining HVM Environment

  - Overview ..............................................................................................................66

  - Accessing environment version............................................................................66

  - Virtual instructions for determining environment................................................66

Figures

#### Figures

Figure 1-1 HVM architecture......................................................................................8 Figure 6-1 Virtual interrupt controller......................................................................24 Figure 9-1 Translation list entry................................................................................33 Figure 9-2 Page table entry (generic)........................................................................34 Figure 9-3 Page directory entry ................................................................................35 Figure 9-4 Page table entry (L1 – 4MB)...................................................................35 Figure 9-5 Page table entry (L1 – 16MB).................................................................35 Figure 9-6 Page table entry (L2)...............................................................................35

Tables

Tables

| Table 1-1 | Terminology...................................................................................................10 |
| --- | --- |
| Table 5-1 | Event registers................................................................................................20 |
| Table 5-2 | GSR field encodings......................................................................................21 |
| Table 5-3 | Event types.....................................................................................................22 |
| Table 5-4 | Virtual instructions for event management....................................................23 |
| Table 6-1 | Virtual instructions for interrupt management...............................................26 |
| Table 7-1 | General exceptions.........................................................................................28 |
| Table 7-2 | Machine check exceptions.............................................................................29 |
| Table 8-1 | HVM cache control operations......................................................................30 |
| Table 8-2 | Virtual instructions for event management....................................................31 |
| Table 9-1 | Translation list entry fields ............................................................................33 |
| Table 9-2 | L1 page table entry types...............................................................................34 |
| Table 9-3 | Page table entry fields....................................................................................36 |
| Table 9-4 | LPN bits used as function of page size..........................................................37 |
| Table 9-5 | Cache attribute types......................................................................................37 |
| Table 9-6 | Virtual instructions.........................................................................................38 |
| Table 10-1 Virtual instructions for processor management .............................................41 Table A-1 HVM virtual instruction summary.................................................................45 Table B-1 Virtual instructions for determining environment .........................................66 |  |

Revision History

#### Revision History

| Revision | Date | Description |
| --- | --- | --- |
| A | July 2011 | Initial version of document; reflects version 0.7 of Hexagon Virtual Machine (HVM). |

- Introduction

1.1	Overview

The Hexagon™ Virtual Machine (HVM) provides system programmers with an abstraction layer which enables multiple operating systems and other clients to execute concurrently on a single Hexagon processor, through the virtualization and partitioning of physical hardware resources.

For example, HVM enables the Hexagon processor to concurrently run the following software systems while enforcing strict resource protection between them:

■ A fine-tuned real-time application managing special devices and services.

■ A general-purpose operating system implementing a user interface and application stack.

HVM virtualizes the Hexagon processor itself as multiple virtual processors which execute concurrently, while ensuring security and quality of service.

1.2	Architecture

Figure 1-1 shows the major functional units of the HVM architecture.

Hexagon Virtual Machine Virtual Interrupt Controller; Logical Memory.. Virtual Virtual Virtual Processor Processor Processor

Virtual Machine Monitor

Hardware

Figure 1-1	HVM architecture

The Hexagon Virtual Machine consists of the virtualized programming environment in which a client software system runs. It includes the following items:

■ One or more virtual processors

■ Virtual interrupt controller

■ Logical memory

The virtual processors (which can be started and stopped) share the virtual interrupt controller and logical memory.

The Virtual Machine Monitor (VMM) is a software layer which manages the hardware resources and provides them to HVM in their virtualized form.

The hardware layer consists of the Hexagon hardware resources. This includes the Hexagon processor and related subsystems (memory, interrupt controller, etc.) which are managed by the VMM.

NOTE	The VMM supports multiple concurrent HVM instances – each instance is referred to as a virtual machine.

1.3	Modes

HVM supports two levels of privilege within the virtual machine:

■ HVM User mode

■ HVM Guest mode

Guest mode is privileged with respect to User mode, but neither mode is privileged to directly access the underlying hardware.

1.4	Instructions and events

HVM provides virtual instructions and events which control the transitions between User and Guest modes, and which enable a guest operating system to manage the privileged virtual resources.

1.5	Virtual privileged resource model

The HVM virtual privileged resource model was designed with the following goals:

■ Expose the minimum amount of detail on the underlying privileged hardware resources.

■ Perform as efficiently as possible when implemented in software on the Hexagon V2 processor and later processor versions.

■ Define interfaces to support their implementation in hardware (either whole or partial) on future Hexagon processor versions.

1.6	Using this manual

This document is intended for system programmers who are targeting their software for operation on an HVM Monitor or Hypervisor. It is also intended as a reference for HVM implementors.

1.7	Terminology

Except where otherwise noted, the terms used to describe HVM are identical to those used to describe the Hexagon processor architecture.

Table 1-1	Terminology

| Term | Definition |
| --- | --- |
| Event record | Data structure passed on the Guest-mode stack during a virtual event, providing context information necessary for the handling of the event. |
| Guest mode | Operating mode under which operating systems run on HVM to provide various services to user applications. |
| GBADVA | Guest Bad Virtual Address |
| GELR | Guest Event Link Register |
| GOSP | Guest Other Stack Pointer |
| GSR | Guest Status Register |
| HVM | Hexagon Virtual Machine Virtualized programming environment in which a client software system runs. It includes one or more virtual processors. logical memory, and a virtual interrupt controller. |
| Logical memory | Virtual machine memory accessible by HVM clients. Logical memory is the memory that is not mapped by the HVM memory management unit (MMU) for use in constructing virtual memory maps in HVM. |
| PTE | Page table entry |
| User mode | Operating mode under which most HVM applications run. |
| Virtual event | Asynchronous invocation of Guest mode by the virtual machine. |
| Virtual instruction | An operation requesting a function of HVM that cannot be expressed by a native Hexagon processor instruction.  Virtual instructions are generally implemented as Hexagon instructions which trap to the underlying hardware Supervisor mode, which performs the necessary function. |
| Virtual machine | HVM instance running on the VMM. |

Table 1-1	Terminology (continued)

| Term | Definition |
| --- | --- |
| Virtual processor | HVM resource that is necessary to run a program.   A virtual machine can support multiple instances of a virtual processor, with each instance executing concurrently and sharing common memory and I/O resources. |
| VMM | Virtual Machine Monitor Software layer which underlies HVM. It manages the hardware resources and provides them to HVM in their virtualized form. |

1.8	Notation

This document uses the same notational conventions used in the Hexagon Programmer’s Reference Manual. For details see the appropriate version of that document.

- HVM User Mode

  - Overview

Except for the restrictions noted below, HVM User mode is identical to the processor architecture defined in the Hexagon Programmer’s Reference Manual.

Most HVM clients run in User mode.

  - Restrictions

HVM User mode has the following restrictions:

■ Memory addressing

#### 2.2.1	Memory addressing

The VMM reserves the high 16 MB of the User-mode address space (addresses 0xff00_0000 to 0xffff_ffff) for its own use.

Any attempts by an HVM client to load, store, or execute from addresses in this range will result in a protection-violation event being posted to Guest mode.

NOTE	This restriction also applies to the Guest-mode address space (Section 3.3.1).

- HVM Guest Mode

3.1	Overview

HVM Guest mode is a superset of HVM User mode (Chapter 2).

In addition to the standard Hexagon processor architecture supported by User mode, Guest mode defines virtual instructions and virtual events to support the implementation of protected, multi-user, multi-tasking operating systems that run on HVM.

Virtual instructions are extensions to the Hexagon instruction set – they invoke HVM operations that cannot be performed by a regular Hexagon instruction. Virtual instructions are generally implemented as Hexagon traps which switch the processor to its Supervisor mode.

Virtual events are asynchronous transfers from User to Guest mode – they provide a controlled way to switch between the HVM modes. Virtual events can be triggered by exceptions or interrupts.

NOTE	HVM virtual instructions execute like regular processor instructions, but are always treated as solo instructions which cannot be grouped in packets with other instructions.



Appendix A provides details on the encoding and semantics of the virtual instructions.

3.2	Guest mode capabilities

HVM Guest mode has the following capabilities:

■ Event handling

■ Interrupts

■ Exceptions

■ Cache control

■ Memory management

■ Processor resource management

#### 3.2.1	Event handling

HVM can transfer control asynchronously to handle events:

■ Handler vectors must be registered using privileged virtual instruction vmsetvec

■ Event handling is terminated with virtual instruction vmrte

For more information see Chapter 5.

#### 3.2.2	Interrupts

HVM supports Hexagon interrupts as a special case of asynchronous events:

■ Interrupts are individually maskable

■ Interrupts can be collectively enabled and disabled

■ Interrupts are managed with virtual instructions vmintop, vmsetie, and vmgetie

For more information see Chapter 6.

#### 3.2.3	Exceptions

HVM supports Hexagon exceptions as another type of event:

■ Exceptions are not maskable (unlike interrupts)

For more information see Chapter 7.

Hexagon Virtual Machine Specification [Draft]

#### 3.2.4	Cache control

HVM supports certain Hexagon cache control operations:

■ Privileged operations (Guest mode only)

■ Managed with virtual instruction vmcache

For more information see Chapter 8.

#### 3.2.5	Memory management

HVM implements a virtual memory management unit:

■ Privileged operations (Guest mode only)

■ Autonomous page table walking

■ Page tables in canonical HVM format

■ Managed with virtual instructions vmnewmap and vmclrmap

For more information see Chapter 9.

#### 3.2.6	Processor resource management

HVM supports virtual processors and a fine-grained timer:

■ Privileged operations (Guest mode only)

■ Create and destroy concurrently-executing virtual processor instances

■ Virtual processors managed with virtual instructions vmstart and vmstop

■ High-precision 64-bit timer

■ Timer managed with virtual instructions vmgettime and vmsettime

For more information see Chapter 10.

3.3	Restrictions

HVM Guest mode has the following restrictions:

■ Memory addressing

#### 3.3.1	Memory addressing

The VMM reserves the high 16 MB of the Guest-mode address space (addresses 0xff00_0000 to 0xffff_ffff) for its own use.

Any attempts by an HVM client to load, store, or execute from addresses in this range will result in a protection-violation event being posted to Guest mode.

NOTE	This restriction also applies to the User-mode address space (Section 2.2.1).

- Initial State

  - Overview

This chapter describes the initial state of the following HVM resources:

■ Virtual processor

■ Registers

■ Memory

  - Virtual processor

A virtual processor begins executing in Guest mode. The virtual MMU is always active.

  - Registers

For virtual processors the initial value of the program counter (PC) and stack pointer are specified externally: either as parameters to commands passed to the VMM, or as arguments to the virtual instruction vmstart which creates a new virtual processor instance (Section 10.4).

The initial values of all other registers are undefined – these registers must be initialized by software starting at the initial PC.

Implementors Note: While register values are undefined at the startup of a virtual processor, if the hardware processor resources are re-assigned, the register values of trusted virtual processors must never be inherited by untrusted virtual processors.

  - Memory

When an initial program is loaded by the VMM, the program image may contain initialized data. Otherwise the initial memory state of HVM consists of undefined memory values.

Implementors Note: While memory values are undefined at startup of a virtual machine, in the event that the hardware resources are re-assigned, memory values of trusted virtual machines must never be inherited by untrusted virtual machines.

Initial State

  - Initial memory map

In the HVM initial memory map, the logical memory segment that the initial program is loaded into is mapped to the identical virtual address range, with consecutive virtual pages mapped 1:1 to consecutive logical pages.

The page tables describing the initial map are outside the initial logical address space, and cannot be modified by HVM clients (Section 2.2.1, Section 3.3.1).

Any other mapping must be created by Guest-mode software constructing a new set of page tables and activating them with virtual instruction vmnewmap (Section 9.5).

- Event Model

  - Overview

HVM supports virtual events (or more simply, events) as a means for transferring control between User-mode applications and Guest-mode operating systems. HVM events can be caused by Hexagon processor exceptions or interrupts.

  - Event model

Events are specified by their assigned event number. When an event occurs, HVM uses the event number as an index into the event vector table to determine which event handler will process the event. The vector table and handlers are defined by the guest operating system.

Before it begins receiving events, the guest operating system must register the Guest Event Vector Base (GEVB) value with HVM. It does this with the virtual instruction vmsetvec.

HVM responds to an event by storing event-related information (called the event record) in the dedicated event registers (Section 5.3), and then transferring control to the event vector specified by the event number.

Event handlers access the event registers with the virtual instructions vmgetregs and vmsetregs – the guest operating system is responsible for managing these registers.

When an event is dispatched to a handler, interrupts are disabled to the virtual processor. Interrupts are re-enabled either explicitly when the handler executes the virtual instruction vmsetie, or implicitly when the handler returns by executing the virtual instruction vmrte. (vmrte restores the interrupt enable value from the event record.)

When HVM transfers control between User and Guest modes, it swaps the contents of the Guest Other Stack Pointer (GOSP) and R29 (the Hexagon processor stack pointer).

If an event handler causes an additional event, the original event record is lost because the event registers are overwritten. Similarly, when returning from an event, the event registers are reloaded by the guest operating system before executing the virtual instruction vmrte. Care must be taken to ensure that no event occurs during this time.

NOTE	If an event occurs before GEVB is configured, HVM terminates the current virtual processor (Section 10.4).

## 5.3	Event registers

Table 5-1 defines the four 32-bit registers that are used to store HVM event records (Section 5.2).

Table 5-1	Event registers

| Register | Alias | Name |
| --- | --- | --- |
| GELR | G0 | Guest Event Link Register |
| GSR | G1 | Guest Status Register |
| GOSP | G2 | Guest Other Stack Pointer |
| GBADVA | G3 | Guest Bad Virtual Address value |

The Hexagon processor instruction set does not provide instructions for directly accessing the event registers. Instead, these registers are accessed with the virtual instructions vmgetregs and vmsetregs.

NOTE	Future Hexagon processor versions may support direct access to the event registers, eliminating the need for these virtual instructions.

### 5.3.1	GELR

GELR is the Guest Event Link Register.

It contains a 32-bit virtual address value specifying where the virtual processor resumes executing after an event is serviced.

If a guest operating system wishes a processed event to return to a different address in User mode (for example, in the case of a user-level context switch), the GELR value can be modified before the virtual instruction vmrte is executed.

### 5.3.2	GSR

GSR is the Guest Status Register.

It provides information about the virtual processor state when the event occurred, and about the nature of the event beyond what is implicitly indicated by the event vector.

| 31 | 30 | 29 |  |  |  |  | 24 | 23 |  |  |  |  |  |  | 16 | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UM | IE |  | Reserved |  |  |  |  |  | Res |  |  | erved |  |  |  |  | Ca |  |  |  |  |  |  | use |  |  |  |  |  |  |  |

Table 5-2 describes the fields of the GSR word.

Table 5-2	GSR field encodings

| Bits | Name | Description |
| --- | --- | --- |
| 31 | UM | Set if virtual processor was executing in User mode when the event was taken, clear otherwise. |
| 30 | IE | Set if virtual processor had interrupts enabled when the event was taken, clear otherwise. |
| 15:0 | Cause | Cause code associated with an Exception or Interrupt event. |

NOTE	The User Mode and Interrupt Enable mask of the virtual processor are restored from GSR by a vmrte instruction.

### 5.3.3	GOSP

GOSP is the Guest Other Stack Pointer.

When executing in User mode, this register contains the Guest-mode stack pointer.

If an event occurs during User-mode execution, HVM swaps the contents of GOSP and R29 (the Hexagon processor stack pointer). Thus when the event vector is reached, R29 contains the Guest-mode stack pointer, and GOSP stores the User-mode stack pointer.

When the virtual instruction vmrte switches from Guest back to User mode, HVM swaps R29 and GOSP again, saving the Guest-mode stack pointer in GOSP and restoring the Usermode stack pointer in R29.

### 5.3.4	GBADVA

GBADVA is the Guest Bad Virtual Address value.

When servicing a memory-related event (page fault, alignment error, permissions violation), GBADVA contains the 32-bit virtual address that caused the event.

## 5.4	Event types

Table 5-3 summarizes the event types.

Table 5-3	Event types

| Event Number | Event Name | Event Description | Notes |
| --- | --- | --- | --- |
| 0 | Reserved |  |  |
| 1 | Machine  Check | Unrecoverable virtual machine state (Section 7.5) | Non-maskable exception. Virtual machine execution terminates if not handled. |
| 2 | General  Exception | Precise internal hardware or software exception (Section 7.3) | Non-maskable exception |
| 3, 4 | Reserved |  |  |
| 5 | Trap0 | SW Trap0 instruction (Section 7.4) | Non-maskable exception |
| 6 | Reserved |  |  |
| 7 | Interrupt | General external interrupts (Chapter 6) | Maskable. Interrupt priority decreases with increasing interrupt number, Interrupt0 being highest. |

NOTE	The Hexagon architecture specifies that if an instruction packet contains multiple exception-causing instructions, then Slot 1 exceptions are resolved before Slot 0 exceptions. If a single instruction has multiple exceptions, the priority is listed in the table from highest (first row) to lowest (last row).

## 5.5	Event vector registration

Guest-mode software can register a service vector for an event by executing the virtual machine instruction vmsetvec, which is passed a 32-bit Guest-mode virtual address.

Events begin executing at the address computed by the following formula:

GEBV + (event_number * 4)

## 5.6	Event handler return

Event handlers return by executing the virtual instruction vmrte. This instruction uses the values stored in the event registers to restore the following states:

■ User/Guest mode and interrupt enable (stored in GSR.UM and GSR.IE)

■ Program counter (stored in GELR)

■ Stack pointer (stored in GOSP) if the value in GSR.UM indicates that the mode switch is from Guest to User

NOTE vmrte assumes that a valid event record is stored in the event registers.

## 5.7	Virtual instructions for event management

Table 5-4 summarizes the virtual instructions used for event management.

Table 5-4	Virtual instructions for event management

| Instruction | Description |
| --- | --- |
| vmsetvec | Register event vector table |
| vmgetregs | Get event register values |
| vmsetregs | Set event register values |
| vmsetie | Enable interrupts |
| vmrte | Return from event servicing |

NOTE	For details on these instructions see Appendix A.

## 5.8	Programmers note

An HVM instance begins executing in Guest mode, so the only way to execute software in User mode is to perform the following steps:

- Set the event registers so the GSR.UM bit is set, and the GELR value specifies code to be executed in User mode.

- Execute a vmrte instruction.

# Interrupts

## Overview

In HVM interrupts are handled as a special case of events. When an interrupt event is dispatched, the GSR.Cause value in the event record is set to the interrupt number.

Interrupts can be enabled or completely disabled per virtual processor by setting the virtual state Interrupt Enabled. When an event is taken, Interrupt Enabled is cleared and the previous Interrupt Enabled state is stored in GSR.

## Virtual interrupt controller

HVM defines a virtual interrupt controller to manage interrupt events flowing into the multiple virtual processors.

| 0 1 2 3 N 0 1 2 3 N 0 1 2 3 Local Enable Global Enable 0 1 2 3 N Pending Interrupts 0 1 2 3 N ... ... ... ... ... 0 1 2 3 N 0 1 2 3 N 0 1 2 3 Local Enable Global Enable 0 1 2 3 N Pending Interrupts 0 1 2 3 N ... ... ... ... ... |  |  |  |  |  | N. |
| --- | --- | --- | --- | --- | --- | --- |
| VP 0 |  | VP 1 | ... | VP N |  |

Figure 6-1	Virtual interrupt controller

Hexagon Virtual Machine Specification [Draft]	Interrupts

Each interrupt has a global enable, as well as a per-virtual processor local enable:

■ If an interrupt is disabled globally, no virtual processor takes the interrupt.

■ If an interrupt is disabled locally, the specific virtual processor does not take the interrupt.

By manipulating the local and global enable values, interrupts can be steered to specific virtual processors, or received by any available virtual processor.

When an interrupt arrives, its bit is set in the pending-interrupt bit vector. If the interrupt is globally and locally enabled for a virtual processor that has interrupts enabled, the virtual processor may take the interrupt: the PC changes to the interrupt event, and the interrupt number is stored in GSR.CAUSE. In addition, the interrupt is cleared from the pending register and automatically disabled globally.

NOTE	At most one virtual processor takes a given interrupt. Interrupting a virtual machine and clearing the pending bit and global enable are atomic operations.

## Interrupt type and polarity

The VMM is responsible for configuring interrupt type and polarity.

## Interrupt masks

HVM has both a per-interrupt global enable bit and a per-interrupt, per-virtual-processor enable bit. These bits can be modified by the virtual instruction vmintop, and can be used to mask interrupts either for a specific processor or for all processors.

## Interrupt acknowledgement

When HVM posts an interrupt event, the interrupt is disabled globally. Thus, any further occurrences of that interrupt are not posted on any virtual processor until the interrupt service routine re-enables it using the GLOBEN operation of the virtual instruction vmintop.

## Interrupt enable

Each virtual processor has an Interrupt Enabled status. This status can be checked using the virtual instruction vmgetie, and modified using the virtual instruction vmsetie (as well as vmrte). When a virtual processor has the Interrupt Enabled status disabled (set to zero), it takes no interrupts.

The Interrupt Enabled status is automatically set to Disabled on every taken event (exception or interrupt), as well as when the virtual processor starts.

Hexagon Virtual Machine Specification [Draft]	Interrupts

## Virtual instructions for interrupt management

Table 6-1 summarizes the virtual instructions used for interrupt management.

Table 6-1	Virtual instructions for interrupt management

| Instruction | Description |  |
| --- | --- | --- |
| vmintop | Perform interrupt-related operation: ■ Globally enable interrupt |  |
|  | ■ | Globally disable interrupt |
|  | ■ | Locally enable interrupt |
|  | ■ | Locally disable interrupt |
|  | ■ | Set interrupt affinity |
|  | ■ | Post software interrupt |
|  | ■ | Check for pending interrupt |
|  | ■ | Take pending interrupt |
|  | ■ | Clear pending interrupt |
|  | ■ | Get interrupt pending, global enable, local enable |
| vmgetie | Return current Interrupt Enabled state |  |
| vmsetie | Set Interrupt Enabled state and return old state |  |

The vmintop operation “Set interrupt affinity” locally enables an interrupt for the specified virtual processor, and locally disables it for all other virtual processors.

NOTE	For details on these instructions see Appendix A.

# Exceptions

## Overview

Exceptions are non-interrupt events, and are not maskable like interrupts.

Exceptions can be precise or imprecise. When a precise exception event is handled, GELR contains the following value:

■ General exceptions – The address of the instruction packet that caused the exception

■ Trap exceptions – The packet following the trap instruction

NOTE	When an exception occurs as a consequence of a load or store address, GBADVA contains the address associated with the exceptional operation.

## Exception classes

HVM supports three classes of exceptions, each of which is handled separately:

■ General exceptions

■ Trap exceptions

■ Virtual machine check

Hexagon Virtual Machine Specification [Draft]	Exceptions

## General exceptions

General exceptions in HVM correspond closely to precise exceptions in the underlying Hexagon processor. The general exception value is stored in the GSR.Cause field of the event record.

Table 7-1	General exceptions

| Exception | Description | GBADVA |
| --- | --- | --- |
| 0x00 | Reserved | Undefined |
| 0x01 | Precise Bus or Memory Error. | Undefined |
| 0x02-0x10 | Reserved | Undefined |
| 0x11 | Execute Protection Violation. Attempt to fetch instructions from virtual memory page without execute permission. | Undefined |
| 0x12-0x13 | Reserved | Undefined |
| 0x14 | User Access Violation. Attempt to fetch instructions in User mode from virtual memory page without user access permission. | Undefined |
| 0x15 | Invalid Instruction Packet. Attempt to execute malformed instruction packet, or one containing reserved or invalid opcodes or combinations of operations. | Undefined |
| 0x16-1A | Reserved | Undefined |
| 0x1B | Privilege Violation. Attempt to execute privileged instruction or virtual instruction in User mode. | Undefined |
| 0-x1C | Misaligned Program Counter. Attempt to transfer control to misaligned instruction packet address. | Undefined |
| 0x1D-1F | Reserved | Undefined |
| 0x20 | Load to Misaligned Address | Misaligned Load Address |
| 0x21 | Store to Misaligned Address | Misaligned Store Address |
| 0x22 | Load Protection Violation. Attempt to load data from virtual memory page without read permission. | Protected Load Address |
| 0x23 | Store Protection Violation. Attempt to store data to virtual memory page without write permission. | Protected Store Address |
| 0x24 | Load User Access Violation. Attempt to load data in User mode from page without user access permission. | Protected Load Address |
| 0x25 | Store User Access Violation. Attempt to store data in User mode to page without user access permission. | Protected Store Address |
| 0x26-27 | Reserved | Undefined |
| 0x28 | Cache Conflict. Instruction packet contains a set of instructions and address modes that are incompatible with cache attributes of referenced addresses. | Undefined |
| 0x29 | Instruction packet with destination register collision | Undefined |
| 0x30-0xFF | Reserved | Undefined |

Exceptions

## Trap exceptions

Trap exceptions are triggered when a trap0 instruction is executed by an HVM client.

The 8-bit value encoded in the trap0 instruction is stored in the GSR.Cause field of the event record.

NOTE	On trap exceptions GELR points to the packet following the trap instruction, and not to the trap instruction itself.

## Virtual machine check

Machine Check exceptions indicate the occurrence of potentially unrecoverable failures in the virtual machine or its underlying hardware.

HVM software can register and handle machine check exceptions to perform emergency shutdown operations or to attempt recovery. Normal termination of a machine check event handler is performed with a vmstop virtual instruction (and not vmrte).

If the Guest-mode software has fully handled the failure (for example, the fault has been isolated to a single User-mode program which can be terminated in isolation), then it can terminate with a vmrte. However, it is implementation-dependent whether the virtual machine monitor will allow resumption of pre-event processing.

Table 7-2	Machine check exceptions

| Exception | Description |
| --- | --- |
| 0x00 | Shutdown. Administrative shutdown of virtual machine |
| 0x01 | Invalid Guest mode stack pointer |
| 0x02 | Reserved |
| 0x03 | Invalid logical page mapping in page table detected at runtime |
| 0x04-0x1F | Reserved |
| 0x20 | Imprecise Data Abort to underlying hardware |
| 0x21-0x2F | Reserved |
| 0x30 | NMI to underlying hardware |
| 0x31-0xFFFF | Reserved |

# Cache Control

## Overview

The user-level Hexagon cache management instructions (dcfetch, icinva, dccleaninva, dccleana, dcinva) can be used in HVM clients.

Additional cache control operations that must be handled by the VMM are performed with the virtual instruction vmcache.

## HVM cache operations

The virtual instruction vmcache is used to perform various cache operations that are difficult (or inefficient) to perform with the user-level Hexagon cache control instructions (dcfetch, icinva, dccleaninva, dccleana, dcinva).

vmcache additionally enables cleaninva-like operations over large virtual address areas to be optimized by the VMM (Section A.2.1).

NOTE vmcache operations are non-destructive of data in the cache.

Table 8-1 lists the HVM-specific cache control operations.

Table 8-1	HVM cache control operations

| Operation | Description |
| --- | --- |
| ICKILL | Non-destructive invalidate of entire instruction cache |
| DCKILL | Non-destructive invalidate of entire data cache |
| L2KILL | Non-destructive invalidate of entire L2 data cache |
| DCCLEANINVA | Non-destructive invalidate of virtual address range, starting at address in R1 for the number of bytes in R2, through full D-cache hierarchy. |
| ICINVA | Non-destructive invalidate of virtual address range, starting at address in R1 for the number of bytes in R2, through full I-cache hierarchy. |
| IDSYNC | Ensure that Instruction cache can observe all data in specified address range. NOTE - Similar to DCCLEANINVA followed by ICINVA. |
| Reserved | Reserved (No-op) |

Cache Control

## Virtual instructions for cache control

Table 8-2 summarizes the virtual instructions used for HVM cache control.

Table 8-2	Virtual instructions for event management

| Instruction | Description |
| --- | --- |
| vmcache | Perform one of several HVM cache control operations |

For details on these instructions see Appendix A.

# Memory Management

## Overview

HVM maps virtual addresses 0x0000_0000 to 0xfeff_ffff onto a 32-bit logical address space, using either a list of translations, or a 1- or 2-level virtual page table scheme. The translations are visible to and writable by a guest operating system running on HVM.

The VMM is responsible for validating and transforming the information as necessary when filling the physical processor TLB. In normal operation, translations are loaded ondemand by the VMM.

## Underlying logical/physical memory

“Logical” addresses are addresses not translated by the HVM MMU. Depending on the virtual machine implementation and configuration, they may be hardware physical addresses or addresses mapped by the VMM.

NOTE	The ranges of logical memory accessible in HVM may be determined by platform-specific operations. Attempts to map and access logical addresses which are not accessible to a virtual machine will result in a protection violation exception event.

## Linear translations

Virtual-to-logical address translations can be specified by a list of translations. Figure 9-1 shows the format of a list entry. Table 9-1 describes the fields in a list entry.

The VMM traverses the translation list in a linear fashion. The list is terminated by a translation entry which has all bits set to zero.

NOTE	This format requires less space to hold simple memory layouts, and may be preferable for more highly-embedded Guest-mode environments.

63	54	5251	32

| L | Reserved |  |  |  |  | Size |  |  |  | Virtual Page |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 31 30 29 28 27 |  |  |  |  | 24 23 2019 |  |  |  |  | 0 |
| X | W | R | U |  | C |  |  | - |  | Logical Page |

Figure 9-1	Translation list entry

Table 9-1	Translation list entry fields

| Field | Description |
| --- | --- |
| Logical Page | Logical Page Number that the corresponding Virtual Page maps to |
| Virtual Page | Virtual Page Number that is matched against the load or store address |
| Size | Page size: ■ 000: 4 KB ■ 001: 16 KB ■ 010: 64 KB ■ 011: 256 KB ■ 100: 1 MB ■ 101: 4 MB ■ 110: 16 MB NOTE - All other values are reserved. Programming an entry with a reserved pattern results in undefined behavior. |
| U, R,W, X | Permissions: ■ U: User-enable. Set if user accesses are allowed for this page ■ R: read-enable. Set if reads are allowed from this page. ■ W: write-enable. Set if writes are allowed to this page. ■ X: execute-enable. Set if execution is allowed for this page. If permissions are violated, a precise exception will be raised with the appropriate cause code (load, store, or fetch). |
| C | Cacheability (Section 9.4.3) |
| L | Link bit. If this bit is set, the current entry points to next set of translation entries. Bits 31:0 specify address of the next translation in the list; the other bits in the entry are ignored. |
| Reserved | Reserved field. Field currently ignored by HVM. Because it may be used in future HVM versions, the recommended value for this field is 0. |

## Virtual page table entries

HVM defines two levels of virtual page table. The first level breaks down the virtual address space into 1020 4MB segments, with each segment represented by a first-level page table entry (PTE).

A first-level PTE always contains the size of the virtual memory page that is mapped:

■ In the case of pages 4MB or greater, the first-level entry contains the translation and permissions information for the page.

■ For pages less than 4MB, the first-level entry contains a pointer to an array of second-level entries.

A first-level PTE encodes the entry type and page size in its 3-bit S field (Figure 9-3). The definition of the remaining bits varies according to the entry type.

31	2	0

|  | S. |
| --- | --- |

Figure 9-2	Page table entry (generic)

Table 9-2 lists the entry types for an L1 page table entry.

Table 9-2	L1 page table entry types

| S Field Value | Entry Type | Page Size | L2 Entries | Address Bits |
| --- | --- | --- | --- | --- |
| 000 | Page Directory Entry | 4KB | 1024 | 31:12 |
| 001 | Page Directory Entry | 16KB | 256 | 31:10 |
| 010 | Page Directory Entry | 64KB | 64 | 31:8 |
| 011 | Page Directory Entry | 256KB | 16 | 31:6 |
| 100 | Page Directory Entry | 1MB | 4 | 31:4 |
| 101 | Page Table Entry | 4MB | N/A | N/A |
| 110 | Page Table Entry | 16MB | N/A | N/A |
| 111 | Invalid | Invalid | N/A | N/A |

If the size specified in S is greater than or equal to 4M, a second level of page table is not necessary to define the mapping: the entry defines the page mapping and permissions directly. For smaller pages, the entry is interpreted as a directory entry which points to a second-level table.

NOTE Referencing a virtual address which resolves to a PTE marked as invalid (111) results in a protection violation exception event.



Because the granularity of the first-level page table is 4MB, the PTE for a 16MB virtual page must be replicated for each of the 4 entries in the firstlevel page table that correspond to the 16MB virtual address range.

### Page directory entries

A PTE contains (in addition to the S field) a pointer to a second-level page table. The most significant bits of the entry are used as the most significant bits of the logical address of the L2 table.

Figure 9-3 shows the format of a page directory entry.

| 31 4 3 2 |  |  | 0 |
| --- | --- | --- | --- |
| L2 Table Address | - |  | S |

Figure 9-3	Page directory entry

NOTE	L2 tables must be aligned to the size of the table.

### Page table entries

PTEs are used to define the virtual-to-logical translation for a specific memory page.

Instead of a pointer to a second-level page table, a PTE can specify a 4MB or 16MB translation at the L1 level. Figure 9-4 and Figure 9-5 show the corresponding entry formats (with the S field set to specific values per Table 9-2).

| 31 22 21 |  | 12 11 10 9 8 |  |  |  |  | 6 5 4 3 2 |  |  |  |  | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Logical Page | - |  | X | W | R |  | C | U | T | - |  | 5 |

Figure 9-4	Page table entry (L1 – 4MB)

| 31 24 23 |  | 12 11 10 9 8 |  |  |  |  | 6 5 4 3 2 |  |  |  |  | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Logical Page | - |  | X | W | R |  | C | U | T | - |  | 6 |

Figure 9-5	Page table entry (L1 – 16MB)

Second-level page tables specify translations for a contiguous 4MB of the address space.

Each entry has the same translation size. This uniform size is specified at the first level. Figure 9-6 shows a second-level PTE.

| 31 | 12 11 10 9 8 |  |  |  |  | 6 5 4 3 2 0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Logical Page |  | X | W | R |  | C | U | T | - | - |

Figure 9-6	Page table entry (L2)

Table 9-3 describes the fields in a PTE.

Table 9-3	Page table entry fields

| Bits | Mnemonic | Description |
| --- | --- | --- |
| 31:12 | LPN | Logical Page Number. High-order bits of page-aligned logical address corresponding to mapped page. The number of bits actually used depends on specified page size.  See Table 9-4. |
| 11 | X | Execute Permission. HVM has execute access to page. Attempts fetch instructions from an address within page causes an Execute Permission Exception event if the X bit is cleared (0). |
| 10 | W | Write Permission HVM has write access to page. Attempts to store data to an address within the page causes a Write Permission Exception event if W bit is cleared (0). |
| 9 | R | Read Permission HVM has read access to page. Attempts to load data from an address within the page causes a Read Permission Exception event if the R bit is cleared (0). |
| 8:6 | C | Cache Attributes See Section 9.4.3. |
| 5 | U | User Mode If the bit is set (1), User mode programs may reference the page. If it is cleared (0), read, write, or execute references in User mode result in a User Access Exception event. |
| 4 | T | Trusted Behavior depends on the system trust model. Recommended setting is cleared (0). |
| Varies | – | Unused. Ignored by HVM. |

NOTE	For L1 entries, the size field must be set correctly for determining page size.

For L2 entries, the size field is ignored and may be used by software.



If none of the R, W, or X bits are set, the virtual map is considered invalid, and any access (load, store, fetch) results in a Page Fault Exception event.

Table 9-4	LPN bits used as function of page size

| Page SIze | LPN Bits Used |
| --- | --- |
| 4KB | 31:12 |
| 16KB | 31:14 |
| 64KB | 31:16 |
| 256KB | 31:18 |
| 1MB | 31:20 |
| 4MB | 31:22 |
| 16MB | 31:24 |

NOTE HVM ignores any LPN bits that are not required to address an aligned page of the specified size.

### Cache attributes

In a page table entry (Section 9.4.2) the C field controls the cache attributes of the virtual page. The cache behavior is undefined if virtual aliases map the same logical page with different cache attributes.

C is relevant only for “terminal” PTEs which resolve to a target logical page address.

Table 9-5	Cache attribute types

| C Field Value | L1 Cache Policy | L2 Cache Policy |
| --- | --- | --- |
| 0 | Write-back | Non-cacheable |
| 1 | Write-through | Non-cacheable |
| 2 | Reserved |  |
| 3 | Reserved |  |
| 4 | Device Access |  |
| 5 | Write-through | Cacheable |
| 6 | Uncached Memory |  |
| 7 | Write-back | Cacheable |
| 8-15 | Reserved |  |

The cache attribute Device Access should be used whenever loads or stores have side effects.

The cache attribute Uncached Memory does not allow memory to be cached. However, the implementation is allowed to replay loads and stores.

Implementors Note: The HVM implementation is free to replay loads and stores for pages assigned Uncached Memory. (The Hexagon processor will in fact do this.) NOTE	To maintain compatibility with future HTM versions, the reserved cache attribute settings should not be used.



HVM may choose to reduce the cacheability of a translation for reasons of correctness or Quality-of-Service.

## Setting new memory maps

The virtual instruction vmnewmap establishes a new logical-to-virtual memory map for HVM.

vmnewmap accepts as arguments the logical addresses of either a segment table

(Section 9.4) or linear translation list (Section 9.3). The instruction immediately following vmnewmap executes with the new memory map in effect.

Implementors Note: If an invalid page table is passed to vmnewmap, the HVM implementation is free to determine whethervmnewmap returns with a value indicating failure, or whether a subsequent attempt to de-reference an address with an invalid mapping causes a virtual machine check. For more information see Section 7.5.

## Flushing stale memory maps

The virtual instruction vmclrmap flushes a possibly-stale virtual mapping.

vmclrmap accepts as arguments a range of virtual addresses. It causes copies of virtual address translations to be purged from the virtual machine (including the TLB) for the specified range of virtual addresses.

Subsequent accesses to the memory range use the values in the memory map tables.

## Virtual instructions for memory management

Table 9-6 summarizes the virtual instructions used for memory management.

Table 9-6	Virtual instructions

| Instruction | Description |
| --- | --- |
| vmnewmap | Register new logical-to-virtual memory map. |
| vmclrmap | Flush a possibly-stale virtual mapping. |

For details on these instructions see Appendix A.

- Processor Resource Management

  - Overview

HVM supports the following facilities for processor resource management:

■ Fine-grained system timer

■ Ability to suspend processor execution

■ Ability to create new virtual processors

  - Timer

HVM supports a fine-grained timer which is accessed through the virtual instructions vmgettime and vmsettime:

■ vmgettime returns a 64-bit value representing the current time.

■ vmsettime resets the timer to the 64-bit value passed as an argument.

Implementors Note: The HVM implementation is free to define whether the timer corresponds to CPU time or wall-clock time. For example, if a Virtual Machine Monitor schedules out a virtual CPU, the timer may or may not continue to increment while the virtual CPU is not executing.

Hexagon Virtual Machine Specification [Draft]	Processor Resource Management

  - Processor suspension

Guest-mode software on HVM can suspend the execution of a virtual processor in two ways:

■ The virtual instruction vmwait

■ The virtual instruction vmyield

vmwait suspends execution pending an interrupt event. After the next event is handled, the virtual instruction vmrte that terminates the event servicing will result in execution resuming at the instruction after the vmwait instruction.

vmyield suspends execution temporarily, independently of any virtual processor events. It auses the VMM to schedule other virtual processors at the same or higher priority, without the need for pre-emption.

NOTE	It is recommended that the guest operating system yield while attempting to acquire a spinlock. This ensures that a virtual processor holding the spinlock has an opportunity to execute.

10.4	Processor creation

Guest-mode software can create new instances of virtual processors with the virtual instruction vmstart. The newly-created instances execute concurrently with the existing processor instances.

A new virtual processor instance begins execution in Guest mode, fetching instructions from the address specified as an argument to vmstart. The new instance begins execution with the same memory map as the virtual processor instance that executed the vmstart.

Each virtual processor has a unique 32-bit identifier, which can have values in the following range:

0 .. (max_number_of_supported_virtual_processors - 1)

A virtual processor ID value can never have the value -1 (all ones). The initial virtual processor instance has a virtual processor ID of zero. Software running on a virtual processor can access its virtual processor ID by executing the virtual instruction vmvpid.

The inverse of vmstart is the virtual instruction vmstop. When a virtual processor executes vmstop, the following events occur:

■ Execution of the virtual processor is terminated.

■ The instruction following the vmstop is not issued.

■ The resources associated with the virtual processor are freed and made available for use by any new virtual processor instance created by a subsequent vmstart operation.

Hexagon Virtual Machine Specification [Draft]	Processor Resource Management

10.5	Virtual instructions for processor management

Table 10-1 summarizes the virtual instructions used for processor resource management.

Table 10-1	Virtual instructions for processor management

| Instruction | Description |
| --- | --- |
| vmgettime | Get the 64-bit virtual processor timestamp |
| vmsettime | Set virtual processor timestamp value. |
| vmwait | Wait on an interrupt event. |
| vmyield | Voluntarily reschedule virtual processor resources. |
| vmstart | Start a new, concurrently executing virtual processor. |
| vmstop | Terminate virtual processor execution. |
| vmvpid | Get virtual processor ID. |

For details on these instructions see Appendix A.

- Trust Model

  - Overview

The HVM environment provides a trust model which is distinct from, but interoperable with the trust model of other cores in the system.

- Debug

  - Overview

HVM debug support must reconcile two factors:

■ The development requirement for simple and effective debugging of programs executing on the virtual machine.

■ The operational requirement for security and isolation.

  - User-mode software debug

User-mode software can be debugged using Guest-mode facilities. For example:

■ Generating page faults on memory references

■ Inserting trap0 instructions for breakpoints and single-step operation

These facilities are sufficient to implement a UNIX-style ptrace capability for the guest operating system.

  - Guest-mode software debug

The VMM can use the Hexagon processor’s supervisor-mode facilities to debug Usermode code. These same process-level facilities can be used to debug Guest-mode software.

Implementors Note: The HVM implementation is free to determine whether the VMM accepts debugging commands directly from some operator communications channel (such as a serial port or a JTAG interface), or whether the VMM provides an API to trusteddebug-agent module which translates from operator/host debugger commands on the communications channel and VMM debug API functions.

Under no circumstances, however, can an untrusted VMM or operator debug agent generate trusted load or store cycles, either directly or indirectly, through a trusted system that is being debugged.

- HVM Instructions

  - Overview

HVM provides virtual instructions and events which control the transitions between User and Guest modes, and which support the implementation of protected, multi-user, multitasking operating systems that run on HVM.

Virtual instructions are extensions to the Hexagon instruction set – they invoke HVM operations that cannot be performed by a regular Hexagon instruction.

This appendix provides detailed descriptions of the HVM virtual instructions. The instructions are listed alphabetically.

Table A-1 summarizes the virtual instructions.

Table A-1	HVM virtual instruction summary

| Mnemonic | Function | Encoding | Inputs | Outputs |
| --- | --- | --- | --- | --- |
| vmversion | Request Virtual Machine Version | trap1(#0) | R0 = Requested VM Version | R0 = Configured VM Version |
| vmrte | Return from Event | trap1(#1) | Event Record in g3-g0 | N/A |
| vmsetvec | Set event vector | trap1(#2) | R0 = Vector Table Address | R0 = 0 on success, otherwise -1 |
| vmsetie | Set interrupt enable state | trap1(#3) | R0 1 to enable, 0 to disable | R0 = Previous enable state in bit 0 |
| vmgetie | Get interrupt enable state | trap1(#4) | N/A | R0 = Enable state in bit 0 |
| vmintop | Interrupt Operation | trap1(#5) | R0 = Interrupt Op  R1-R4: Depends on Op | R0 depends on Op. |
| vmclrmap | Clear virtual memory map | trap1(#10) | R0 = Starting VA R1 = Length in bytes | R0 = 0 on success, otherwise -1 |
| vmnewmap | Set new virtual memory map | trap1(#11) | R0 =Logical address of new segment table R1 =Type of translations | R0 = 0 on success, otherwise negative error code |
| vmcache | Virtual Machine cache control | trap1(#13) | R0 = Operation to be performed R1 = Starting virtual address R2 = Length in bytes | R0 = 0 on success, otherwise -1 |
| vmgettime | Get Virtual Machine Time | trap1(#14) | N/A | R0 = least significant 32 bits of timestamp R1 = most significant 32 bits of timestamp |
| vmsettime | Set Virtual Machine time | trap1(#15) | R0 = least significant 32 bits of timestamp R1 = most significant 32 bits of timestamp | N/A |
| vmwait | Wait for next interrupt | trap1(#16) | N/A | R0 = Interrupt number of reactivating event |
| vmyield | Voluntarily reschedule virtual processor | trap1(#17) | N/A | N/A |
| vmstart | Create new virtual processor instance | trap1(#18) | R0 = Starting execution address R1 = Starting stack pointer | R0 = Virtual processor number of new virtual processor on success, otherwise -1 |
| vmstop | Terminate current virtual processor instance | trap1(#19) | N/A | N/A |
| vmvpid | Obtain virtual processor ID | trap1(#20) | N/A | R0 = Virtual processor number of virtual processor executing the instruction |
| vmsetregs | Set Guest Registers | trap1(#21) | R0-3 hold G0-3 values | N/A |
| vmgetregs | Read Guest Registers | trap1(#22) | N/A | R0-3 hold G0-3 values. |

  - Instruction properties

HVM virtual instructions have the following properties:

■ They are privileged, and can be executed only in Guest-mode software. If executed in User mode, they will generate a privilege violation exception.

■ By convention they take their inputs (if any) from the Hexagon processor registers R0–R4, and return their outputs in registers R0 and R1, as necessary.

■ They are solo instructions (as defined in the Hexagon Programmer’s Reference Manual), and thus cannot be grouped with other instructions in an instruction packet.

■ They are generally implemented as Hexagon traps which switch the processor to its Monitor mode.

#### A.2.1	VMCACHE

Perform cache maintenance operations which are either impossible or inefficient using the native cache management instructions.

Syntax

vmcache

Registers

| Register | In/Out | Description |
| --- | --- | --- |
| R0 | In | Cache operation to perform: #0: ICKILL - Nondestructive Invalidate of entire instruction cache #1: DCKILL - Nondestructive invalidate of entire data cache #2: L2KILL - Nondestructive invalidate of entire L2 data cache #3: DCCLEANINVA - Nondestructive invalidate of the range of virtual addresses starting at the address in R1 for the number of bytes in R2 through the full D-cache hierarchy. #4: ICINVA - Nondestructive invalidate of the range of virtual addresses starting at the address in R1 for the number of bytes in R2 through the full I-cache hierarchy. #5: IDSYNC - Ensures that Instruction cache can observe all data in the specified range; similar to DCCLEANINVA followed by ICINVA. other: Reserved (No-Op) |
|  | Out | 0: Operation successful -1: Operation unsuccessful |
| R1 | In | Starting virtual address for range-based operations |
| R2 | In | Byte count for range-based operations |

Type: JR (slot 2)

Exceptions

■ Store protection exception on range-based operations if virtual address range contains pages without write permission.

■ Privilege violation exception if executed in User mode

Notes

■ This is a solo instruction. It must not be grouped with other instructions in a packet.

Encoding

| 31 |  |  |  |  |  |  |  | 23 | 22 |  |  |  |  |  | 16 | 15 | 14 | 13 | 12 |  |  |  | 8 | 7 |  | 5 | 4 |  | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | VM (trap1) |  |  |  |  |  |  |  |  | - |  |  |  |  |  | Parse |  | - | VM Group 1 |  |  |  |  | - |  |  | vmcache |  |  | - |  |
| 0 | 1 | 0 | 1 | 0 | 1 | 0 | 0 | 1 | - | - | - | - | - | - | - | P | P | - | 0 | 0 | 0 | 0 | 1 | - | - | - | 1 | 0 | 1 | - | - |

#### A.2.2	VMCLRMAP

Flush possible stale virtual mapping.

Syntax

vmclrmap

Registers

| Register | In/Out | Description |
| --- | --- | --- |
| R0 | In | Starting virtual address. Purge MMU state for the virtual address range starting at the address in R0, and covering the number of bytes in R1. NOTE - This would typically follow a modification to an active page table. |
|  | Out | 0: Operation successful -1: Operation unsuccessful |
| R1 | In | Byte count |

Type: JR (slot 2)

Type

■ Privilege violation exception if executed in User mode

Notes

■ This is a solo instruction. It must not be grouped with other instructions in a packet.

Encoding

| 31 |  |  |  |  |  |  |  | 23 | 22 |  |  |  |  |  | 16 | 15 | 14 | 13 | 12 |  |  |  | 8 | 7 |  | 5 | 4 |  | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | VM (trap1) |  |  |  |  |  |  |  |  | - |  |  |  |  |  | Parse |  | - | VM Group 1 |  |  |  |  | - |  |  | vmclrmap |  |  | - |  |
| 0 | 1 | 0 | 1 | 0 | 1 | 0 | 0 | 1 | - | - | - | - | - | - | - | P | P | - | 0 | 0 | 0 | 0 | 1 | - | - | - | 0 | 1 | 0 | - | - |

#### A.2.3	VMGETIE

Get virtual processor interrupt enable value.

Syntax

vmgetie

Registers

| Register | In/Out | Description |
| --- | --- | --- |
| R0 | Out | 0: Interrupts disabled on virtual processor 1: Interrupts enabled on virtual processor |

Type: JR (slot 2)

Type

■ Privilege violation exception if executed in User mode

Notes

■ This is a solo instruction. It must not be grouped with other instructions in a packet.

Encoding

| 31 |  |  |  |  |  |  |  | 23 | 22 |  |  |  |  |  | 16 | 15 | 14 | 13 | 12 |  |  |  | 8 | 7 |  | 5 | 4 |  | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | VM (trap1) |  |  |  |  |  |  |  |  | - |  |  |  |  |  | Parse |  | - | VM Group 0 |  |  |  |  | - |  |  | vmgetie |  |  | - |  |
| 0 | 1 | 0 | 1 | 0 | 1 | 0 | 0 | 1 | - | - | - | - | - | - | - | P | P | - | 0 | 0 | 0 | 0 | 0 | - | - | - | 1 | 0 | 0 | - | - |

#### A.2.4	VMINTOP

Operate on virtual interrupt controller.

Syntax

vmintop

Registers

| Register | In/Out | Description |
| --- | --- | --- |
| R0 | In | Interrupt operation to perform: 0: VMINTOP_NOP: Do nothing 1: VMINTOP_GLOBEN: Global Enable 2: VMINTOP_GLOBIDIS: Global Disable 3: VMINTOP_LOCEN: Local Enable 4: VMINTOP_LOCDIS: Local Disable 5: VMINTOP_AFFINITY: Set Interrupt Affinity 6: VMINTOP_GET: Take Next Interrupt 7: VMINTOP_PEEK: Query which interrupt is next, without taking 8: VMINTOP_STATUS: For an interrupt, return pending and enablement status 9: VMINTOP_POST: Post an interrupt into interrupt controller 10: VMINTOP_CLEAR: Clear an interrupt without taking it |
|  | Out | Non-zero value for failure for operations 1-5 and 9-10. R0=6:GET: returns 0 if interrupt taken, -1 if not taken. R0=7: PEEK: returns highest priority pending/enable interrupt, otherwise -1. R0=8: STATUS: 0 if pending, 1 if local enable. 2: global enable. -1 on failure. |
| R1 | In | Interrupt that the operation acts upon. |
| R2 | In | Only used if R0 = 5: the virtual processor to take the interrupt. |

Type: JR (slot 2)

Type

■ Privilege violation exception if executed in User mode

Notes

■ This is a solo instruction. It must not be grouped with other instructions in a packet.

Encoding

| 31 |  |  |  |  |  |  |  | 23 | 22 |  |  |  |  |  | 16 | 15 | 14 | 13 | 12 |  |  |  | 8 | 7 |  | 5 | 4 |  | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | VM (trap1) |  |  |  |  |  |  |  |  | - |  |  |  |  |  | Parse |  | - | VM Group 0 |  |  |  |  | - |  |  | vmintop |  |  | - |  |
| 0 | 1 | 0 | 1 | 0 | 1 | 0 | 0 | 1 | - | - | - | - | - | - | - | P | P | - | 0 | 0 | 0 | 0 | 0 | - | - | - | 1 | 0 | 1 | - | - |

#### A.2.5	VMGETTIME

Get 64-bit virtual processor timestamp.

Syntax

vmgettime

Registers

| Register | In/Out | Description |
| --- | --- | --- |
| R0 | Out | Least-significant 32 bits of virtual processor timestamp |
| R1 | Out | Most-significant 32 bits of virtual processor timestamp |

Type: JR (slot 2)

Type

■ Privilege violation exception if executed in User mode

Notes

■ This is a solo instruction. It must not be grouped with other instructions in a packet.

Encoding

| 31 |  |  |  |  |  |  |  | 23 | 22 |  |  |  |  |  | 16 | 15 | 14 | 13 | 12 |  |  |  | 8 | 7 |  | 5 | 4 |  | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | VM (trap1) |  |  |  |  |  |  |  |  | - |  |  |  |  |  | Parse |  | - | VM Group 1 |  |  |  |  | - |  |  | vmgettime |  |  | - |  |
| 0 | 1 | 0 | 1 | 0 | 1 | 0 | 0 | 1 | - | - | - | - | - | - | - | P | P | - | 0 | 0 | 0 | 0 | 1 | - | - | - | 1 | 1 | 0 | - | - |

#### A.2.6	VMNEWMAP

Establish new logical-to-virtual memory map.

The virtual machine’s MMU is reprogrammed to discard the previous virtual-physical mappings in effect prior to the vmnewmap instruction and use the mapping described by the page tables whose segment table (L1 page table) begins at the address in R0. If the translations map logical addresses that are outside the virtual machine’s legal logical address space, it is implementation specific whether the operation fails, returning a nonzero error code, or whether a machine check event will be generated when an illegal entry is encountered at run time. VMM implementations may not implement all translation table types. An unsupported translation type will result in an error and translations will not be changed.

Syntax

vmnewmap

Registers

| Register | In/Out | Description |
| --- | --- | --- |
| R0 | In | Logical address of new MMU segment table. |
|  | Out | 0 if operation was successful, otherwise an implementation-specific negative error code. |
| R1 | In | 0: Linear list of translations 1: Set of page tables |

Type: JR (slot 2)

Type

■ Privilege violation exception if executed in User mode

■ Machine check event if invalid page table subsequently detected at run time by a virtual machine implementation which does not do full page table validation at vmnewmap time.

Notes

■ This is a solo instruction. It must not be grouped with other instructions in a packet.

■ The virtual machine instruction following the vmnewmap operation will be the instruction at the address following the vmnewmap’s address in the original map, as translated by the new map. Care must be taken in replacing the mapping of guest OS pages containing vmnewmap instructions.

Encoding

| 31 |  |  |  |  |  |  |  | 23 | 22 |  |  |  |  |  | 16 | 15 | 14 | 13 | 12 |  |  |  | 8 | 7 |  | 5 | 4 |  | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | VM (trap1) |  |  |  |  |  |  |  |  | - |  |  |  |  |  | Parse |  | - | VM Group 1 |  |  |  |  | - |  |  | vmnewmap |  |  | - |  |
| 0 | 1 | 0 | 1 | 0 | 1 | 0 | 0 | 1 | - | - | - | - | - | - | - | P | P | - | 0 | 0 | 0 | 0 | 1 | - | - | - | 0 | 1 | 1 | - | - |

#### A.2.7	VMRTE

Return from event service

■ PC = G0 (GELR)

■ Interrupt Enable State = G1[30] (GSR.IE)

■ User Mode = G1[31] (GSR.UM)

■ if (User Mode) swap(R29,G2) (GOSP)

Syntax

vmrte

Registers

| Register | In/Out | Description |
| --- | --- | --- |
| R29 | In/Out | Stack pointer that may be swapped with GOSP |

Type: JR (slot 2)

Type

■ Privilege violation exception if executed in User mode

Notes

■ This is a solo Instruction. It must not be grouped with other instructions in a packet.

Encoding

| 31 |  |  |  |  |  |  |  | 23 | 22 |  |  |  |  |  | 16 | 15 | 14 | 13 | 12 |  |  |  | 8 | 7 |  | 5 | 4 |  | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | VM (trap1) |  |  |  |  |  |  |  | - |  |  |  |  |  | Parse |  | - | VM Group 0 |  |  |  |  | - |  |  | vmrte |  |  | - |  |
| 0 | 1 | 0 | 1 | 0 | 1 | 0 | 0 | 1 | - | - | - | - | - | - | - | P | P | - | 0 | 0 | 0 | 0 | 0 | - | - | - | 0 | 0 | 1 | - | - |

#### A.2.8	VMSETIE

Set virtual processor interrupt enable value.

Syntax

vmsetie

Registers

| Register | In/Out | Description |
| --- | --- | --- |
| R0 | In | 1 (or any odd value): Enable interrupts to the virtual processor 0 (or any even value): Disable interrupts to the virtual processor |
|  | Out | 0: Interrupts were previously disabled on the virtual processor 1: Interrupts were previously enabled on the virtual processor |

Type: JR (slot 2)

Type

■ Privilege violation exception if executed in User mode

Notes

■ This is a solo instruction. It must not be grouped with other instructions in a packet.

Encoding

| 31 |  |  |  |  |  |  |  | 23 | 22 |  |  |  |  |  | 16 | 15 | 14 | 13 | 12 |  |  |  | 8 | 7 |  | 5 | 4 |  | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | VM (trap1) |  |  |  |  |  |  |  |  | - |  |  |  |  |  | Parse |  | - | VM Group 0 |  |  |  |  | - |  |  | vmsetie |  |  | - |  |
| 0 | 1 | 0 | 1 | 0 | 1 | 0 | 0 | 1 | - | - | - | - | - | - | - | P | P | - | 0 | 0 | 0 | 0 | 0 | - | - | - | 0 | 1 | 1 | - | - |

#### A.2.9	VMSETTIME

Set virtual processor timestamp value.

Syntax vmsettime

Registers

| Register | In/Out | Description |
| --- | --- | --- |
| R0 | In | Least-significant 32 bits of virtual processor timestamp |
| R1 | In | Most-significant 32 bits of virtual processor timestamp |

Type: JR (slot 2)

Type

■ Privilege violation exception if executed in User mode

Notes

■ This is a solo Instruction. It must not be grouped with other instructions in a packet.

Encoding

| 31 |  |  |  |  |  |  |  | 23 | 22 |  |  |  |  |  | 16 | 15 | 14 | 13 | 12 |  |  |  | 8 | 7 |  | 5 | 4 |  | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | VM (trap1) |  |  |  |  |  |  |  |  | - |  |  |  |  |  | Parse |  | - | VM Group 1 |  |  |  |  | - |  |  | vmsettime |  |  | - |  |
| 0 | 1 | 0 | 1 | 0 | 1 | 0 | 0 | 1 | - | - | - | - | - | - | - | P | P | - | 0 | 0 | 0 | 0 | 1 | - | - | - | 1 | 1 | 1 | - | - |

#### A.2.10	VMSETVEC

Set event handler vector table for events.

Syntax

vmsetvec

Registers

| Register | In/Out |  | Description |
| --- | --- | --- | --- |
| R0 | In | Vector table address |  |
|  | Out | 0: Success -1: Failure |  |

Type: JR (slot 2)

Type

■ Privilege violation exception if executed in User mode

Notes

■ This is a solo Instruction. It must not be grouped with other instructions in a packet.

Encoding

| 31 |  |  |  |  |  |  |  | 23 | 22 |  |  |  |  |  | 16 | 15 | 14 | 13 | 12 |  |  |  | 8 | 7 |  | 5 | 4 |  | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | VM (trap1) |  |  |  |  |  |  |  |  | - |  |  |  |  |  | Parse |  | - | VM Group 0 |  |  |  |  | - |  |  | vmsetvec |  |  | - |  |
| 0 | 1 | 0 | 1 | 0 | 1 | 0 | 0 | 1 | - | - | - | - | - | - | - | P | P | - | 0 | 0 | 0 | 0 | 0 | - | - | - | 0 | 1 | 0 | - | - |

#### A.2.11	VMSTART

Start a new, concurrently-executing virtual processor.

If resources are available, a new virtual processor is allocated to begin execution in guest mode with the instruction address contained in R0. The stack pointer (R29) of the new virtual processor is set to the value of R1 of the virtual processor executing the vmstart. The virtual processor number of the new virtual processor is returned in R0 of the virtual processor executing the vmstart.

If resources are unavailable, the operation fails, and a value of -1 is returned in R0.

Syntax

vmstart

Registers

| Register | In/Out | Description |
| --- | --- | --- |
| R0 | In | Virtual address of first instruction to be executed by new virtual processor |
|  | Out | If successful, the unique identifier number of the newly created virtual processor. If unsuccessful, a value of -1. |
| R1 | In | Virtual address of initial top of stack for new virtual processor |

Type: JR (slot 2)

Type

■ Privilege violation exception if executed in User mode

Notes

■ This is a solo instruction. It must not be grouped with other instructions in a packet.

■ If the initial instruction virtual address is not valid, an execute protection violation exception event is taken by the new virtual processor.

Encoding

| 31 |  |  |  |  |  |  |  | 23 | 22 |  |  |  |  |  | 16 | 15 | 14 | 13 | 12 |  |  |  | 8 | 7 |  | 5 | 4 |  | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | VM (trap1) |  |  |  |  |  |  |  |  | - |  |  |  |  |  | Parse |  | - | VM Group 2 |  |  |  |  | - |  |  | vmstart |  |  | - |  |
| 0 | 1 | 0 | 1 | 0 | 1 | 0 | 0 | 1 | - | - | - | - | - | - | - | P | P | - | 0 | 0 | 0 | 1 | 0 | - | - | - | 0 | 1 | 0 | - | - |

#### A.2.12	VMSTOP

Terminate virtual processor execution.

The virtual processor executing the vmstop instruction ceases execution, and its resources are made available for future vmstart operations.

Syntax

vmstop

Type: JR (slot 2)

Type

■ Privilege violation exception if executed in User mode

Notes

■ This is a solo instruction. It must not be grouped with other instructions in a packet.

Encoding

| 31 |  |  |  |  |  |  |  | 23 | 22 |  |  |  |  |  | 16 | 15 | 14 | 13 | 12 |  |  |  | 8 | 7 |  | 5 | 4 |  | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | VM (trap1) |  |  |  |  |  |  |  |  | - |  |  |  |  |  | Parse |  | - | VM Group 2 |  |  |  |  | - |  |  | vmstop |  |  | - |  |
| 0 | 1 | 0 | 1 | 0 | 1 | 0 | 0 | 1 | - | - | - | - | - | - | - | P | P | - | 0 | 0 | 0 | 1 | 0 | - | - | - | 0 | 1 | 1 | - | - |

#### A.2.13	VMVERSION

Request and obtain the virtual machine version

Syntax

vmversion

Registers

| Register | In/Out | Description |
| --- | --- | --- |
| R0 | In | Requested version for HVM to conform to |
|  | Out | Virtual machine version that is conformed to |
| R1 | In | Most-significant 32 bits of virtual processor timestamp |

Type: JR (slot 2)

Type

■ Privilege violation exception if executed in User mode

Notes

■ This is a solo instruction. It must not be grouped with other instructions in a packet.

■ The version corresponding to this spec is 0x0000_0700.

Encoding

| 31 |  |  |  |  |  |  |  | 23 | 22 |  |  |  |  |  | 16 | 15 | 14 | 13 | 12 |  |  |  | 8 | 7 |  | 5 | 4 |  | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | VM (trap1) |  |  |  |  |  |  |  |  | - |  |  |  |  |  | Parse |  | - | VM Group 0 |  |  |  |  | - |  |  | vmversion |  |  | - |  |
| 0 | 1 | 0 | 1 | 0 | 1 | 0 | 0 | 1 | - | - | - | - | - | - | - | P | P | - | 0 | 0 | 0 | 0 | 0 | - | - | - | 0 | 0 | 0 | - | - |

#### A.2.14	VMVPID

Get virtual processor identification number of the current processor.

Syntax

vmvpid

Registers

| Register | In/Out | Description |
| --- | --- | --- |
| R0 | Out | Virtual processor identification number of current processor |

Type: JR (slot 2)

Type

■ Privilege violation exception if executed in User mode

Notes

■ This is a solo instruction. It must not be grouped with other instructions in a packet.

Encoding

| 31 |  |  |  |  |  |  |  | 23 | 22 |  |  |  |  |  | 16 | 15 | 14 | 13 | 12 |  |  |  | 8 | 7 |  | 5 | 4 |  | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | VM (trap1) |  |  |  |  |  |  |  |  | - |  |  |  |  |  | Parse |  | - | VM Group 2 |  |  |  |  | - |  |  | vmvpid |  |  | - |  |
| 0 | 1 | 0 | 1 | 0 | 1 | 0 | 0 | 1 | - | - | - | - | - | - | - | P | P | - | 0 | 0 | 0 | 1 | 0 | - | - | - | 1 | 0 | 0 | - | - |

#### A.2.15	VMWAIT

Wait on an interrupt event.

The virtual processor issuing the vmwait suspends execution until a configured and unmasked interrupt request is presented to the virtual processor. If interrupts are disabled, execution resumes with the instruction following the vmwait. If interrupts are enabled, the interrupt is serviced with the GELR value in the event record containing the address of the instruction following the vmwait, so that normal execution resumes after a vmrte is executed with the interrupt event record on the top of the stack. In either case, when execution resumes, R0 contains the interrupt number of the interrupt which terminated the wait. If more than one configured and unmasked interrupt is detected before execution resumes, it is implementation dependent which of the interrupts will be represented in R0.

Syntax

vmwait

Registers

| Register | In/Out | Description |
| --- | --- | --- |
| R0 | Out | Interrupt number associated with the interrupt request that terminated the vmwait. |

Type: JR (slot 2)

Type

■ Privilege violation exception if executed in User mode

Notes

■ This is a solo Instruction. It must not be grouped with other instructions in a packet.

Encoding

| 31 |  |  |  |  |  |  |  | 23 | 22 |  |  |  |  |  | 16 | 15 | 14 | 13 | 12 |  |  |  | 8 | 7 |  | 5 | 4 |  | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| VM (trap1) |  |  |  |  |  |  |  |  | - |  |  |  |  |  |  | Parse |  | - | VM Group 2 |  |  |  |  | - |  |  | vmwait |  |  | - |  |
| 0 | 1 | 0 | 1 | 0 | 1 | 0 | 0 | 1 | - | - | - | - | - | - | - | P | P | - | 0 | 0 | 0 | 1 | 0 | - | - | - | 0 | 0 | 0 | - | - |

#### A.2.16	VMYIELD

Voluntarily reschedule virtual processor resources.

If other virtual processors at the same or higher priority are runnable but waiting on processor resources, suspend execution of the issuing virtual processor and schedule the virtual processor with the highest priority.

Syntax

vmyield

Type: JR (slot 2)

Exceptions

■ Privilege violation exception if executed in User mode

Notes

■ This is a solo instruction. It must not be grouped with other instructions in a packet.

Encoding

| 31 |  |  |  |  |  |  |  | 23 | 22 |  |  |  |  |  | 16 | 15 | 14 | 13 | 12 |  |  |  | 8 | 7 |  | 5 | 4 |  | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| VM (trap1) |  |  |  |  |  |  |  |  | - |  |  |  |  |  |  | Parse |  | - | VM Group 2 |  |  |  |  | - |  |  | vmyield |  |  | - |  |
| 0 | 1 | 0 | 1 | 0 | 1 | 0 | 0 | 1 | - | - | - | - | - | - | - | P | P | - | 0 | 0 | 0 | 1 | 0 | - | - | - | 0 | 0 | 1 | - | - |

- Determining HVM Environment

  - Overview

Guest-mode software can poll the VMM for the current HVM version to determine if they are version-compatible.

  - Accessing environment version

When Guest-mode software begins execution, it may need an efficient way to determine the HVM environment provided by the VMM. This can be done with the virtual instruction vmversion, which returns a value specifying the virtual machine environment version that is supported by the VMM.

This enables the guest software to request a virtual machine environment version that it has been verified to be compatible with.

This feature enables the VMM to support guests that may only support an older HVM specification. Alternatively, it enables guests to support Monitors that only support older HVM specifications.

NOTE	Further communication about the system configuration will likely be beneficial for the guest, but the definition of this feature is TBD.



The version number associated with the current VM specification is 0x0000_0700.

B.3	Virtual instructions for determining environment

Table B-1 summarizes the virtual instructions used for determining the HVM environment.

Table B-1	Virtual instructions for determining environment

| Instruction | Description |
| --- | --- |
| vmversion | Return the virtual machine version |

For details on these instructions see Appendix A.